Full-scale Structural Performance Evaluation: State-of-the-Art Test Methods

Ajay Chourasia1, 2*[0000-0001-6595-2465], Alpana Chauhan1 and Moka Eswar1, 2[0000-0003-1235-7341]

1 CSIR - Central Building Research Institute, Roorkee 247667, India

2 Academy of Scientific and Innovative Research (AcSIR), Ghaziabad 201002, India
ajayc@cbri.res.in

Abstract. In the recent years, there is a paradigm shift in research interest to investigate the response of buildings from component-level to system-level, preferably on full-scale when subjected to cyclic lateral loading. Full-scale testing of buildings provides detailed understanding about the behaviour of structural system at global as well as local level. Static, quasi-static, pseudo-dynamic and shake table tests are the commonly adopted experimental techniques for full-scale testing of building systems. However, all these tests have their own merits and constraints in terms of test set-up, time, cost etc. Although ASTM E2126 provides test protocol for such investigation, there is a need for standardization while implementing these techniques. With this viewpoint, an attempt is made to present a standard test-setup for quasi-static assessment of a three-storey building. Test arrangements along with the lateral loading protocol for the proposed quasi-static conditions have also been discussed. Furthermore, this paper critically reviews the existing codal provisions for full-scale testing of buildings, along with the literature on shake table and pseudo-dynamic tests, thereby addressing the gap between guidelines and carried out experiments. The paper illustrates the minimum criteria for conducting full-scale tests on building system, best practices to be adopted for conducting such test, test protocol, response measurement etc, and evaluation of structural performance parameters.

Keywords: Full-scale testing; quasi-static test; pseudo-dynamic test; shake table test; loading protocols; RCC building.

# Introduction

The evaluation of structural performance through full-scale testing of buildings has gained significant traction in recent years, addressing the limitations inherent in component-level testing. Full-scale testing provides a complete understanding of the structural behaviour under real-world conditions. Unlike component-level testing, which isolates individual elements of a structure, full-scale testing captures the interactions and combined effects of all components, thereby offering a more accurate representation of a building's performance. Studies have shown that component-level testing may lead to misleading results due to the effect of boundary conditions and how different parts of a structure interact under load. For instance, Boughton et al. [1] highlighted that component tests do not always take into account the complexities of load paths and stress distributions present in an actual building. Similarly, [2] emphasized that full-scale tests are indispensable for understanding failure mechanisms that cannot be observed in isolated component tests.

Full-scale testing encompasses several methods, each designed to simulate different aspects of real-world loading conditions. Static testing involves applying gradually increasing loads to assess the structure's ultimate load-bearing capacity and deformation characteristics. Quasi-static testing, on the other hand, applies loads in a slow, controlled manner to simulate seismic loading conditions, capturing the structure's response over time. Pseudo-dynamic testing combines aspects of both static and dynamic tests, using computer simulations to apply realistic earthquake loads in a step-by-step manner. Finally, shake table tests involve mounting a full-scale structure on a large platform that can simulate ground motions experienced during an earthquake. Each of these methods has distinct advantages and limitations. Static tests are straightforward and relatively inexpensive but do not capture dynamic effects. Quasi-static and pseudo-dynamic tests offer a more realistic simulation of seismic conditions but require sophisticated equipment and are more time-consuming. Shake table tests provide the most realistic earthquake simulation but are the most costly and complex to perform.[3, 4]

Codal provisions play a crucial role in standardizing full-scale testing methods and ensuring consistent and reliable results. The ASTM E2126 standard [5], for example, outlines protocols for cyclic (reversed) loading, providing guidelines for test setups and load protocols. However, there are gaps in the current codal provisions that need addressing, especially concerning the testing of multi-storey buildings. Studies pointed out that tests on single or two-storey buildings often overlook the complexities introduced by boundary conditions, leading to incomplete assessments of structural performance [6, 7]. Furthermore, the lack of standardized test setups and protocols across different testing facilities complicates the comparison of results and the development of universal design guidelines.

However, conducting full-scale tests presents numerous challenges that researchers must navigate to obtain accurate and reliable results. This paper aims to provide a comprehensive understanding to conduct a full-scale testing of building, addressing the gaps in existing guidelines and providing a framework for future research. By establishing clear protocols for test arrangements, loading conditions, and response measurements, we can enhance the reliability and applicability of full-scale testing outcomes, ultimately contributing to safer and more resilient building designs [8, 9].

# Full-Scale Testing Method

Full-scale structural testing methods are critical for understanding the behaviour of buildings under various load conditions. Each method offers unique insights and presents its own set of advantages and limitations. This section delves into the details of the four primary full-scale testing methods: static testing, quasi-static testing, pseudo-dynamic testing, and shake table testing.

Static testing involves applying loads gradually and steadily to a structure until failure occurs. This method helps in understanding the ultimate load-bearing capacity and the deformation characteristics of the structure. Static tests are straightforward to conduct and require less complex loading equipment compared to dynamic testing setups, making them a cost-effective option. Moreover, static testing provides comprehensive data on the load-deformation behaviour of the structure, which is essential for understanding material properties and structural performance [10]. However, static tests do not account for dynamic effects such as those experienced during earthquakes or wind loads, which limits their applicability in understanding behaviour under such conditions. The gradual application of loads can also be time-consuming, particularly for large or complex structures, and static testing may not reveal certain failure mechanisms that occur under dynamic loading conditions [4, 11].

Fig. 1. Four actuator loading set-up on top storey of a Static Full-scale Test [4]

Quasi-static testing is designed to simulate seismic loading conditions by applying loads in a slow, controlled manner. This method captures the structure’s response over time, making it valuable for seismic performance evaluation. Quasi-static tests can closely simulate the conditions experienced during an earthquake, including cyclic loading patterns, and provide detailed insights into the hysteretic behaviour of structures, which is crucial for understanding energy dissipation and the potential for plastic deformation [12]. The controlled application of loads allows for precise measurement of the structure’s response to cyclic loading. However, quasi-static tests are also time-consuming due to the slow application of loads, which can extend the duration of the testing process. The setup for quasi-static testing is also more complex than static testing, requiring advanced equipment to apply and control the cyclic loads, and they do not fully replicate the real-time dynamic effects of an earthquake [13]. A displacement-controlled quasi-static loading applied at both the first and second-story levels is used by Chourasia et al. [3] as shown in Fig. 2. The loading was conducted in cycles with increasing amplitudes, in accordance with ASTM E2126, to observe the structural behaviour under simulated earthquake conditions. One of the primary challenges encountered during their test was managing the load distribution and ensuring accurate simulation of in-situ conditions, such as the proper connection between precast components.

Fig. 2. Quasi-static Test set-up for a  two storey “3-S” precast building [3]

Pseudo-dynamic (PsD) testing combines aspects of static and dynamic tests, using computer simulations to apply realistic earthquake loads in a step-by-step manner, allowing for the assessment of the structure’s response under simulated seismic conditions. Pseudo-dynamic testing provides a more realistic simulation of seismic events compared to static or quasi-static tests and allows for detailed analysis of the structure’s response at each stage of loading. This method captures both the static load-deformation characteristics and the dynamic effects, providing a comprehensive understanding of structural behaviour [14–16]. In an extensive full-scale pseudo-dynamic test, Chrysostomou et al. [14] evaluated a four-story reinforced concrete frame retrofitted with RC infilling at the European Laboratory for Structural Assessment (ELSA). The PsD tests aimed to assess the effectiveness of seismic retrofitting by transforming selected bays into RC walls. One of the significant challenges was setting up the actuators and aligning the load application uniformly across the structure. The structure was subjected to controlled displacement loads, applied via actuators on each floor. The actuators were braced against a reaction wall, and displacement control was achieved through linear encoders. Ensuring stability during testing was critical due to the stiffness of the RC walls, resulting in difficulties with eigenfrequency response at higher modes. The tests provided valuable insights into the seismic retrofitting of RC frames, with the specimen performing well under varying seismic forces, though some column lap-splice failures and minor cracks were observed. This study highlighted the need for comprehensive seismic design approaches that account for both structural and non-structural components to ensure overall building safety during earthquakes.

Fig. 3. Schematic of the pseudo-dynamic test method [17]

Shake table testing involves mounting a full-scale structure on a large platform that can simulate ground motions experienced during an earthquake. This method provides the most realistic simulation of seismic conditions, including the effects of ground motion and dynamic loading. Shake table tests offer the most comprehensive data on the structure’s performance under real-time dynamic loading, including failure mechanisms and dynamic response characteristics [6, 8, 18–20]. The dynamic behaviour of several ten-story reinforced concrete buildings subjected to seismic loading has been extensively investigated through shake table testing and numerical simulations at the E-Defense research centre in Japan, as summarized by Isakovic et al. [21]. One notable study by Kang et al. [22] involved a series of tests using a three-dimensional shake table to examine both base-slip and fixed-base configurations, aiming to assess the effects of base isolation and foundation rigidity on seismic performance. The total height of the building was 27.45 m, and the typical floor dimensions were 13.5 m x 9.5 m. The structural system consisted of RC walls till the seventh floor, and RC beams and columns for the entire height of the building. In a similar effort, [23] investigated the structural response of another ten-story building, focusing on the behavior under varying seismic excitations and the resulting damage patterns, which highlighted the critical importance of dynamic interactions between structural components. As shown in Fig. 4, the joint on the fourth storey is fractured while the shear cracks are more on lower levels.   These studies collectively provide valuable insights into the seismic performance of high-rise reinforced concrete structures, underscoring the importance of experimental validation in enhancing the accuracy of numerical models and improving structural safety.

Fig. 4. Damage conditions of the specimen building after the 60%-amplitude experiment of the conventional seismic structure [22]

When comparing these methods, it is important to consider the specific objectives of the testing and the available resources. Static testing is suitable for understanding basic load-bearing capacities and deformation characteristics but falls short in simulating dynamic effects. Quasi-static and pseudo-dynamic tests bridge the gap by introducing controlled cyclic loading, providing insights into seismic performance, but at the cost of increased complexity and duration. Shake table testing, while offering the most realistic seismic simulation, is often prohibitively expensive and requires specialized facilities. Understanding these nuances is essential for selecting the appropriate testing method to achieve accurate and reliable results in the evaluation of building performance. As the field continues to evolve, there is a growing need for standardization in test setups and protocols to ensure consistency and comparability across different studies and facilities.

# Difficulties faced during the Full-Scale Testing

Studies have consistently highlighted the difficulties in maintaining synchronized actuator movements, which can lead to significant errors in the results if not properly managed. For instance, Nakashima [24] documented issues with actuator lag during pseudo-dynamic tests on steel structures, where even minor delays caused unintended dynamic effects, compromising test accuracy. These issues were further explored by Bonnet et al. [25] in hybrid simulations, where nonlinear structural behavior exacerbated synchronization problems, requiring advanced multi-loop control systems for real-time adjustments. Chae et al. [18] and [14] also reported significant difficulties in achieving actuator synchronization during hybrid testing of steel frames, with errors leading to inconsistencies between simulated and observed structural responses. Chen H. C. et al. [16] reported that installing and synchronizing the three actuators at each floor level (with capacities of 1000 kN and ±500 mm stroke) was particularly challenging, especially given the need for precise control to simulate realistic seismic conditions, as illustrated in Fig. 5. Despite these challenges, few researchers implemented a predictive control algorithm that adjusted actuator commands in real-time based on feedback from displacement sensors, significantly improving synchronization accuracy.

Fig. 5. Plan and elevation of the full-scale composite frame specimen for pseudo dynamic testing with nine actuators [16]

Accurately simulating boundary conditions in full-scale testing is a significant challenge that can drastically affect the validity of the test results. Boundary conditions dictate how forces are transferred through the structure and how different components interact, making their accurate replication crucial for realistic simulations. In a study by Foti et al. [26], researchers highlighted the complexities involved in modeling boundary conditions, especially in dynamic tests where the interaction between the structure and the ground plays a critical role in seismic performance. Any misrepresentation of these conditions can lead to inaccurate predictions of structural behavior, particularly in terms of load distribution and failure mechanisms. For instance, Pinto et al. [27] discussed the difficulties in simulating soil-structure interaction during pseudo-dynamic testing of bridge piers, where improper boundary conditions resulted in discrepancies between observed and predicted responses. These challenges underscore the need for advanced modeling techniques and careful calibration to ensure that boundary conditions in full-scale tests accurately reflect real-world scenarios, thus providing reliable data for the assessment and design of earthquake-resistant structures.

Maintaining control system stability during full-scale testing is essential to ensure that the applied loads and displacements accurately reflect the intended test conditions. Instability in the control system can lead to errors in force application, inaccurate displacement measurements, and even test failure. Bonnet et al. [25] explored the challenges of maintaining stability in control systems during real-time hybrid testing, where the interaction between physical and numerical substructures required precise timing and synchronization. The study implemented a multi-loop control strategy to manage the complex dynamics involved, though achieving stable and accurate control remained a significant challenge. Similarly, Christenson et al. [28] identified control system instability as a major issue in large-scale experiments, where the nonlinear behavior of the structure under load introduced unexpected variations that could destabilize the system. To address these challenges, researchers have developed adaptive control algorithms that continuously monitor and adjust system parameters in real-time, ensuring that the testing process remains stable and reliable even under the most demanding conditions.

# Codal Provisions for Full Scale Testing

Codal provisions play a crucial role in standardizing full-scale testing methods, ensuring consistent and reliable results that can be universally applied across different testing facilities. The ASTM E2126 [5] standard is one of the primary guidelines for cyclic (reversed) loading, offering detailed protocols for test setups and load applications.. It prescribes detailed steps for applying loads in a controlled manner, specifying load sequences, rates, and measurement techniques to capture the full range of structural behaviour during testing. FEMA 356 [29] and ASCE 41 [30] focus on the seismic evaluation and retrofit of existing buildings, offering comprehensive guidelines for conducting full-scale tests to assess a structure's performance and identify potential weaknesses. These codes emphasize the importance of simulating in-situ conditions, including realistic boundary conditions and load paths, to ensure that the test results accurately reflect the building's likely performance during an earthquake. Both FEMA 356 and ASCE 41 also recommend using performance-based evaluation criteria, which involve assessing the structure's ability to meet specific performance objectives under the simulated loading conditions, such as life safety and collapse prevention, during full-scale tests.

IS 456:2000 [31] focuses on the quality and strength of concrete and reinforcement materials, mandating detailed procedures for testing structural elements to assess their performance under various loading conditions. For full-scale testing, it requires adherence to specified load application methods and careful observation of structural responses, particularly in concrete and steel behaviour. IS 13920:2016 [32] emphasizes the ductile detailing of reinforced concrete structures to enhance their seismic performance. During full-scale seismic tests, this code insists on the proper detailing of reinforcements and the accurate replication of boundary conditions to prevent brittle failure and ensure the structural system's energy dissipation capacity. IS 1893:2016 [33] provides criteria for earthquake-resistant design, including guidelines for simulating seismic forces during full-scale tests. This includes the requirement to replicate the dynamic characteristics of the structure accurately, such as mass distribution and stiffness, to ensure that the test results are representative of actual seismic performance.

# Conclusion

The comprehensive evaluation of full-scale structural performance testing methods underscores the critical importance of such assessments in understanding the real-world behaviour of buildings under seismic loads. This paper reviewed various full-scale testing methods, including static, quasi-static, pseudo-dynamic, and shake table tests, each providing unique insights into the structural and non-structural responses to different loading conditions. The shake table tests, in particular, highlighted the necessity of replicating realistic earthquake motions to capture the dynamic interactions within a building system, as demonstrated in the detailed case studies of the ten-storey structures and the tests conducted at the NEES facility in San Diego. The results from these studies, such as those by Iskovic et al. [21] and [7], reveal significant data on inter-storey drifts, base shear forces, and the impact of floor accelerations and deformations, contributing to improved seismic design and safety standards.

Despite the advancements, current codal provisions, such as ASTM E2126, have gaps that need to be addressed, particularly regarding multi-storey building testing and the complexities of boundary conditions. The need for standardized test methods and protocols is evident to ensure consistency and comparability of results across different testing facilities. The difficulties in managing load distribution, maintaining system stability, and ensuring the integrity of boundary conditions during these tests highlight the need for continuous advancements in testing methodologies and equipment. Future work should focus on refining these methods and expanding the scope of full-scale tests to include a broader range of building types and conditions, ultimately advancing our understanding and mitigation of earthquake risks.

# References

1. 	Boughton GN, Parackal K, Satheeskumar N, Henderson DJ (2017) Development of a Full-Scale Structural Testing Program to Evaluate the Resistance of Australian Houses to Wind Loads. Front Built Environ 3:. https://doi.org/10.3389/fbuil.2017.00021

2. 	McCrum DP, Williams MS (2016) An overview of seismic hybrid testing of engineering structures. Eng Struct 118:240–261. https://doi.org/10.1016/j.engstruct.2016.03.039

3. 	Chourasia A, Kajale Y, Singhal S, Parashar J (2020) Seismic performance assessment of two‐storey precast reinforced concrete building. Struct Concr 21:2011–2027. https://doi.org/10.1002/suco.201900146

4. 	Kabeyasawa T, Mukai T, Fukuyama H, et al (2017) Full Scale Static Loading Test on a Five Story Reinforced Concrete Building (Part2: Damage Analysis). 16th World Conf Earthq Eng

5. 	ASTM E2126-19 (2019) Standard Test Methods for Cyclic (Reversed) Load Test for Shear Resistance of Vertical Elements of the Lateral Force Resisting Systems for Buildings

6. 	Fathali S, Lizundia B (2011) Evaluation of current seismic design equations for nonstructural components in tall buildings using strong motion records. Struct Des Tall Spec Build 20:30–46. https://doi.org/10.1002/tal.736

7. 	Beyer K, Tondelli M, Petry S, Peloso S (2015) Dynamic testing of a four-storey building with reinforced concrete and unreinforced masonry walls: prediction, test results and data set. Bull Earthq Eng 13:3015–3064. https://doi.org/10.1007/s10518-015-9752-z

8. 	Sato E, Furukawa S, Kakehi A, Nakashima M (2011) Full‐scale shaking table test for examination of safety and functionality of base‐isolated medical facilities. Earthq Eng Struct Dyn 40:1435–1453. https://doi.org/10.1002/eqe.1097

9. 	Gavridou S, Wallace JW, Nagae T, et al (2017) Shake-Table Test of a Full-Scale 4-Story Precast Concrete Building. I: Overview and Experimental Results. J Struct Eng 143:1–13. https://doi.org/10.1061/(ASCE)ST.1943-541X.0001755

10. 	Malesza J, Miedzialowski C, Ustinovichius L (2017) Tests on full-scale and static analysis models of the wood-framed building stucture horizontaly loaded. J Civ Eng Manag 23:814–835. https://doi.org/10.3846/13923730.2017.1319411

11. 	Savino P, Tondolo F, Sabia D, et al (2023) Large-Scale Experimental Static Testing on 50-Year-Old Prestressed Concrete Bridge Girders. Appl Sci 13:834. https://doi.org/10.3390/app13020834

12. 	Di Sarno L, Manfredi G (2012) Experimental tests on full‐scale RC unretrofitted frame and retrofitted with buckling‐restrained braces. Earthq Eng Struct Dyn 41:315–333. https://doi.org/10.1002/eqe.1131

13. 	Gupta A, Krawinkler H (1999) Seismic Demands for Performance Evaluation of Steel Moment Resisting Frame Structures

14. 	Chrysostomou CZ, Poljansek M, Kyriakides N, et al (2013) Pseudo-Dynamic Tests on a Full-Scale Four-Storey Reinforced Concrete Frame Seismically Retrofitted with Reinforced Concrete Infilling. Struct Eng Int 23:159–166. https://doi.org/10.2749/101686613X13439149156831

15. 	Negro P, Bournas DA, Molina FJ (2013) Pseudodynamic tests on a full-scale 3-storey precast concrete building: Global response. Eng Struct 57:594–608. https://doi.org/10.1016/j.engstruct.2013.05.047

16. 	Chen CH, Lai WC, Cordova P, et al (2004) Pseudo-Dynamic Test of Full-Scale Rcs Frame: Part I - Design, Construction, Testing. In: Structures 2004. American Society of Civil Engineers, Reston, VA, pp 1–15

17. 	Carrion JE, Spencer BF (2007) Model-based Strategies for Real-time Hybrid Testing. Newmark Struct Eng Lab 6:1–211

18. 	Chae Y, Ricles JM, Sause R (2014) Large‐scale real‐time hybrid simulation of a three‐story steel frame building with magneto‐rheological dampers. Earthq Eng Struct Dyn 43:1915–1933. https://doi.org/10.1002/eqe.2429

19. 	Tura C, Aydın S, Yazgan U, et al (2023) Critical issues in nonlinear modeling of reinforced concrete buildings: code-compliant modeling approaches and the 2015 E-defense shake table tests on a full-scale 10-story building. Bull Earthq Eng 22:329–364. https://doi.org/10.1007/s10518-023-01772-1

20. 	Tondelli M, Petry S, Peloso S, Beyer K (2013) Shake-table test on a four-storey structure with reinforced concrete and unreinforced masonry walls. Proc Vienna Congr Recent Adv Earthq Eng Struct Dyn 2013:28–30

21. 	Isaković T, Tabata K, Wallace J, Polese M (2023) International joint research on the full-scale ten-story RC buildings tested at E-Defense shaking table. Bull Earthq Eng 21:6499–6510. https://doi.org/10.1007/s10518-023-01808-6

22. 	Kang J-D, Kajiwara K, Nagae T, et al (2023) Full-scale shaking table experiments on the seismic response of a 10-story reinforced-concrete building: overview and analysis. Bull Earthq Eng 21:6511–6534. https://doi.org/10.1007/s10518-023-01735-6

23. 	Janevski A, Kang J-D, Isaković T (2023) Simulation of the E-Defense 2015 test on a 10-storey building using macro-models. Bull Earthq Eng 21:6553–6584. https://doi.org/10.1007/s10518-023-01734-7

24. 	Nakashima M (2001) Development, potential, and limitations of real–time online (pseudo–dynamic) testing. Philos Trans R Soc London Ser A Math Phys Eng Sci 359:1851–1867. https://doi.org/10.1098/rsta.2001.0876

25. 	Bonnet PA, Lim CN, Williams MS, et al (2007) Real‐time hybrid experiments with Newmark integration, MCSmd outer‐loop control and multi‐tasking strategies. Earthq Eng Struct Dyn 36:119–141. https://doi.org/10.1002/eqe.628

26. 	Foti S, Passeri F, Rodriguez-Marek A (2019) Uncertainties and variabilities in seismic ground response analyses. In: Earthquake Geotechnical Engineering for Protection and Development of Environment and Constructions, 1st Editio. CRC Press, p 20

27. 	Pinto A V., Pegon P, Magonette G, Tsionis G (2004) Pseudo‐dynamic testing of bridges using non‐linear substructuring. Earthq Eng Struct Dyn 33:1125–1146. https://doi.org/10.1002/eqe.393

28. 	Christenson R, Lin YZ, Emmons A, Bass B (2008) Large-Scale Experimental Verification of Semiactive Control through Real-Time Hybrid Simulation. J Struct Eng 134:522–534. https://doi.org/10.1061/(ASCE)0733-9445(2008)134:4(522)

29. 	FEMA 356 (2000) Prestandard And Commentary For The Seismic Rehabilitation Of Buildings. Federal Emergency Management Agency Washington, D.C

30. 	ASCE/SEI 41-06 (2007) Seismic Rehabilitation of Existing Buildings. American Society of Civil Engineers, Reston, VA

31. 	IS 456 (2000) Plain Concrete and Reinforced

32. 	IS 13920:2016 (2016) Ductile Design and Detailing of Reinforced Concrete Structures Subjected to Seismic Forces - Code of Practice

33. 	IS 1893 (Part 1) : 2016 (2016) Criteria for Earthquake Resistant Design of Structures