3D Numerical Analysis and Optimization of Piled-Raft Foundations

Gaurav Pandey1, Vishal Kumar Mourya1*, Dhirendra Patel1, and Rajesh Kumar1

1Department of Civil Engineering, IIT (BHU), Varanasi-221005, Uttar Pradesh, India

*vishalkmourya.rs.civ18@iitbhu.ac.in

Abstract. A comprehensive 3D numerical model for a pile-cum-raft foundation was developed in the present study. The model considered the complex interactions between piles, the raft, and the soil medium. The model was validated against existing results in the literature, ensuring its reliability. Key parameters influencing the settlements were systematically investigated, including the geometrical characteristics of the foundation elements, such as length, diameter, and number of piles and the thickness of the raft.  Additionally, important soil characteristics such as cohesion and internal friction angle were considered. The soil continuum was modelled using the Mohr-Coulomb failure criterion. For comparative purposes, a raft foundation was also modelled and analysed. The results of this analysis provide valuable information for the economic design of foundations, highlighting the influence of different parameters on settlement behaviour. These findings can aid in optimizing pile-cum-raft foundations for practical engineering applications.

Keywords: Piled-raft foundation, numerical modelling, soil-foundation interaction, Mohr-Coulomb model

- 1 Introduction

As skyscrapers began to be constructed worldwide in the 1970s, many were built on shallow foundations, leading to settlement levels that often exceeded those permitted by design standards. Katzenbach [1] documented such excessive settlements, noting that the Frankfurt Buro Centre building settled by  with a rotation of  degrees, while the Twin Towers of the Deutsche Bank building experienced settlements ranging from  to . To address these challenges, engineers focused on controlling settlements and rotations, minimizing bending moments and sectional forces on ground slabs, and reducing impacts on nearby structures. Although deep foundations, such as pile foundations, offered some solutions, the large number of piles required and the associated costs made this approach uneconomical and overly conservative.

In response, piled raft foundations have gained recognition as a cost-effective and robust alternative for supporting large-scale structures, particularly where shallow foundations alone are insufficient. This hybrid system combines the strengths of both piles and rafts, enhancing structural stability while optimizing material use and reducing costs. The piled raft concept allows the raft to bear a significant portion of the load, with piles strategically employed to control settlements and provide additional support in areas with weaker soil conditions.

The concept of combining piles with rafts was first introduced by Zeevaert [2]. This idea sparked the development of various analytical methods in subsequent years, with notable contributions from Butterfield [3], Poulos [4], Hooper [5] and Burland [6]. Among these, Randolph's approach [7] is particularly well-regarded. Over the last few decades, there has been significant progress in understanding piled raft behaviour through both experimental and numerical analyses. Design methodologies have increasingly sought to account for the complex interactions within piled raft systems [8-9]. Numerical methods, including the Finite Difference Method (FDM), Finite Element Method (FEM), Boundary Element Method (BEM), or their combinations, have been extensively used in this field [5],[10-12]. Experimental studies have also contributed significantly, utilizing 1g tests [13-14] and centrifuge tests [15] to further explore the behaviour of piled raft foundations.

A key observation from the literature is that piled rafts significantly reduce the number or length of piles required compared to traditional pile groups, leading to more economical construction. This approach also enhances the service life of the raft by minimizing both total and differential settlements. Moreover, the optimal arrangement of piles beneath the raft helps to decrease bending moments and internal stresses.

Recent advancements in numerical modelling have enabled more refined analyses of piled rafts. Viggiani et al. [16] emphasized the importance of validating numerical models with field data or experimental results to ensure their accuracy and reliability. Advancements in high-processing computational systems and the development of various commercial codes have made numerical methods the most popular approach for structural analysis. Among these, 3D finite element (FE) analysis stands out as a versatile tool for complex problem-solving. 3D-FE models can account for the non-linear behaviour of soil, the interaction effects between the piles and the raft, and even the influence of soil layering, making them invaluable for designing foundations in complex geotechnical environments. However, its implementation demands significant computational power and expertise in using specialized codes such as PLAXIS, FLAC, and ABAQUS. These codes have been extensively used in recent years to study piled-raft foundation models and their complex interactions.

The present paper provides a 3D numerical analysis of piled-raft foundations using ABAQUS, a finite element program developed by Dassault Systèmes. ABAQUS was chosen over other geotechnical software due to its versatility across various engineering disciplines, its ability to comprehensively assess superstructure-soil interactions, and its advanced features crucial for accurately simulating geotechnical behaviours. The model's validity was confirmed through experimental results found in the literature [17], focusing on the interactions between piles, raft, and soil.

The study explored the effects of several parameters on settlement, including the geometrical properties of the foundation elements (pile length, diameter, and number), raft thickness, and soil characteristics (cohesion and internal friction angle). The findings from these investigations could potentially contribute to the economical design of foundations. The 3D model comprised three main components: the soil continuum, the raft, and a group of piles. The soil was represented as a large cuboid block, with the raft and piles modelled together and embedded into the soil medium. The Mohr-Coulomb model, a plastic model known for its simplicity and empirical validation, was employed to simulate soil behaviour.

Piled-raft foundations, which distribute load between the raft and piles, have become a preferred choice for high-rise buildings due to their load-sharing efficiency and successful applications in countries like Germany, the UAE, and Japan. This paper's investigations aim to contribute to the optimization and economical design of such foundations.

- 2 Model Validation

Validation is crucial in ensuring the accuracy and reliability of finite element analysis (FEA) models in simulating actual behaviour of physical system. It often draws upon information from the literature and laboratory tests for the properties used in the numerical validation. In this section, a three-dimensional model of a piled raft foundation embedded in soil is developed, using geometry and material properties from Vakili [17]. Table 1 details the model's geometry, and Table 2 lists the material properties. The model's results were compared with experimental data to validate its accuracy.

Table 1. Geometry of the model (Vakili, 2015).

| Pile configuration | Size of soil continuum (mm) | Raft dimensions (mm) | Square pile dimensions (mm) |
| --- | --- | --- | --- |
| PR_ |  |  |  |

Table 2. Material properties for the model (Vakili, 2015).

| Pile configuration | Size of soil continuum (mm) | Raft dimensions (mm) | Square pile dimensions (mm) |
| --- | --- | --- | --- |
| Elastic modulus |  |  |  |
| Poisson’s ratio | 0.25 | 0.3 | 0.3 |
| Density | 1554 | 7800 | 7800 |
| Load | Self-weight | Self-weight | Self-weight |

Fig. 1 3D-view of displacement contours (Validated model)

Fig. 2 Model Validation

Equivalent cross-sectional areas show minimal variation in numerical response, making it efficient to use simpler geometries [18]. Therefore, square piles were used instead of circular ones in the simulation. A numerical model of the soil and a  piled raft foundation was developed. Each component was assigned its material properties and assembled with the supporting soil. The Young’s modulus of elasticity for the soil Esand (kPa) was determined using the following expression [19]:

(1)

denotes the adjusted blow count observed using the Standard Penetration Test. Existing literature indicates that the elastic modulus and Poisson’s ratio fell within the range of 10 - 50MPa and 0.25-0.40, respectively. Therefore, adopting the corresponding values of 13MPa and 0.25 for the study is reasonable. Fig. 1 displays the three-dimensional displacement views for a specific case. Fig. 2 compares the experimental results with numerical results from both existing literature and the developed model, using linear-elastic and non-linear plastic models. The comparison shows strong consistency, particularly with the non-linear analysis, which aligns more closely with the experimental results, indicating that non-linear analysis yields more accurate outcomes.

- 3 Parametric analysis of piled-raft foundation

Piled raft foundations involve complex interactions among design parameters like pile arrangement, raft dimensions, soil characteristics, and load conditions. Parametric analysis offers a systematic method to explore various parameter combinations, aiming to optimize foundation performance while ensuring safety and stability.

Table 3 outlines the wide range of parameters investigated in this study. These parameters include geometric properties like pile length, diameter, number of piles, and raft thickness, as well as soil characteristics such as elastic modulus, friction angle, and Poisson’s ratio. The table presents baseline values that remain constant while varying each parameter individually. Throughout the parametric analysis, the soil dimensions were consistently set to 25m length, 25m width, and 30m height. Meanwhile, the raft dimensions were fixed at 5m length, 5m width, with systematic variations in thickness.

Table 3. Variation of parameters used in present study.

| Parameters | Variations | Nominal value |
| --- | --- | --- |
| Pile length |  |  |
| Pile diameter |  |  |
| Number of piles |  |  |
| Raft thickness |  |  |
| Elastic modulus |  |  |
| Angle of internal friction |  |  |
| Poisson’s ratio |  |  |

Fig. 3 Schematic diagram of piled-raft foundation

In this parametric analysis of piled raft foundations, a quarter model is used to study behaviour with circular piles (Fig. 3). The study incorporates soil non-linearity via the Mohr-Coulomb model, capturing diverse soil conditions to comprehensively assess foundation interactions. This model ensures accurate representation of soil complexities and foundation performance across scenarios, following established procedures.

Fig. 4 Meshing used in (a) Soil continuum and (b) Piled-raft

Boundary conditions were carefully assigned to ensure precise simulation within the ABAQUS framework. Along the line of symmetry, the face conditions were specified to maintain stability: Y SYMM (U2 = UR1 = UR3 = 0) and XSYMM (U1 = UR2 = UR3 = 0), signifying symmetry about the corresponding face. Beyond the structure’s periphery, the outer soil face was assigned Displacement/Rotation type boundary conditions, effectively restricting movement in lateral directions. Finally, at the model’s base, fixed boundary conditions were imposed to signify complete immobilization, indicative of no movement or deformation. A gravity and pressure loading step was created to consider self-weight of the system and any external pressure exerted upon it. The separate representations of the piled raft and the soil continuum can be observed in Fig. 4 (a), while Fig. 4 (b) illustrate their assembly.

The meshing was done properly creating partitioning wherever necessary. A meshing strategy (by size) was employed in the lateral direction with element sizes ranging from 15(minimum) to 30(maximum) in lateral direction, with a finer mesh surrounding the piled raft and a coarser mesh in areas farther from it. In the vertical direction, the element size was set at 15 unit. Circular piles were simulated using hexahedral elements and the medial axis algorithm for accurate representation of their geometry. The simulation employed a local seed by number approach, considering circular piles with a total of 8 elements positioned around the circle. A reference settlement of 40mm was fixed using tie constraints to assess bearing capacity under varying geometric and soil conditions.

Finally, the influence of these parameters on the performance of the piled raft was evaluated by analysing load-settlement curves. Load-settlement curves are used in piled raft foundation analysis to assess foundation performance under varying loads, determine bearing capacity, and optimize design by studying settlement behaviour and comparing different foundation configurations. In this context, ‘settlement’ refers to the vertical displacement of the raft relative to the underlying supporting soil.

- 3.1 Effect of length of piles

Longer piles generally contribute to increased load-carrying capacity. As the pile length increases, the additional embedded length provides greater resistance against vertical loads, consequently indicating higher stiffness and enhanced load-bearing capability. Pile length also plays a role in controlling settlement. Longer piles can limit the magnitude of settlement by offering increased support to the foundation. This effect is evident in the gradual progression of the load-displacement curve, demonstrating reduced settlement as pile length increases.

Fig. 5 illustrates the fluctuation in pile length across values of 10m, 12m, 15m, 18m, and 20m. It is evident that as the pile length extends from 10m to 20m, there is a noticeable enhancement in load-bearing capacity, amounting to an approximate 60% increase.

- 3.2 Effect of diameter of piles

Larger diameter piles offer increased surface area in contact with the soil, leading to greater mobilization of soil resistance. This, in turn, results in enhanced load-carrying capacity. As pile diameter increases, the foundation can support higher vertical loads.

Fig. 5 Effect of length of piles

Fig. 6 Effect of diameter of piles

Similar conclusions are drawn from the study (Fig. 6), which illustrates variations in pile diameters, including 0.25m, 0.50m, 0.75m, and 1m. It is evident that considering a diameter of 0.25m as a reference, there is an observable percentage increase in load-bearing capacity with each subsequent 0.25m increment. These increments result in approximate increases of 83%, 200%, and 315%, respectively. Nevertheless, as the diameter increases by 0.25m in each step, the subsequent rise in the percentage increase diminishes, initially 83%, followed by 64%, and ultimately 38%. This suggests that an increase in diameter remains reasonable up to a certain point, beyond which further increments in diameter would yield negligible gains.

Although larger diameter piles provide advantages in relation to load-carrying capacity and stiffness, they might pose construction difficulties. These larger piles necessitate greater excavation and handling efforts, potentially resulting in extended construction duration and escalated costs.

- 3.3 Effect of friction angle

The angle of internal friction significantly influences two key aspects of piles: skin friction along the shaft and end-bearing capacity at the base. Higher friction angles enhance both skin friction and end-bearing capacity, boosting the load-carrying potential of piled raft foundations. Load-settlement curves with higher friction angles demonstrate greater load-bearing capacities and possibly reduced settlements under similar loads due to increased stiffness. Higher friction angles correspond to increased ultimate bearing capacities, affecting the shape and slope of load-settlement curves.

In this study, the angle of internal friction was varied systematically: 25°, 30°, 35°and 40° (Fig. 7). An incremental increase from 25°to 40°led to noticeable improvements in load-bearing capacity. Specifically, capacity increased by 10% from 25°to 30°, and significantly by 16.5% and 21.5% at 35°and 40°, respectively. Notably, the rate of improvement declined as friction angles increased, with subsequent increments showing enhancements of 10%, 5.5%, and 4.4%. This trend aligns with findings in existing literature [18].

- 3.4 Effect of elastic modulus of soil

A higher elastic modulus signifies stiffer soil, leading to more balanced load distribution between the raft and piles. This distribution is critical in determining the contribution of each component to supporting the structure. Additionally, the soil’s elastic modulus affects the extent of settlement experienced by the foundation-stiffer soil tends to exhibit reduced settlement due to its enhanced resistance to deformation.

Fig. 7 Effect of friction angle

Fig. 8 Effect of elastic modulus of soil

Fig. 8 depicts the variation in soil’s elasticity modulus, spanning from 15MPa to 50MPa. As the modulus progresses in 5MPa increments, a concurrent improvement in load capacity is evident. Notably, the load capacity experiences significant percentage enhancements at each step: 16%, 10.35%, 7%, 6.6%, 5.5%, 4.5%, and 4.3%. It is worth noting that this upward trend tends to stabilize as the soil attains a specific level of stiffness.

- 3.5 Effect of Poisson’s ratio

Poisson’s ratio, representing the ratio of lateral strain to axial strain, directly affects the deformation behaviour of soil and the foundation’s response to applied loads. As Poisson’s ratio increases, the lateral expansion of the soil under load becomes more pronounced, impacting the distribution of stresses within the foundation system. Consequently, altering Poisson’s ratio leads to distinctive load-settlement curves, showcasing varying degrees of settlement under the same load increments.

In this ongoing study, Poisson’s ratio is routinely varied across a spectrum spanning from 0.25 to 0.40, covering specific values of 0.25, 0.30, 0.35, and 0.40 (Fig. 9). The results reveal that load capacity likewise exhibits an increase as Poisson’s ratio rises. However, within this range, the observed percentage rise in load capacity fluctuates modestly, ranging between approximately 1.5% and 2.5%. This suggests that Poisson’s ratio exerts a comparatively milder influence compared to other influential parameters in the context of load-settlement curves for piled-raft foundations.

- 3.6 Effect of number of piles

The number of piles in a piled raft foundation directly impacts its load-carrying capacity and settlement behaviour. Increasing the number of piles generally enhances the foundation’s overall capacity due to increased load distribution and reduced pile loads. With an increase in the number of piles, load distribution becomes more uniform across the foundation, reducing individual pile loads. This improved load-sharing capability directly contributes to higher load-carrying capacities.

Fig. 9 Effect of Poisson’s ratio

Fig. 10 Effect of number of piles

This effect is evident in load-settlement curves (Fig. 10) of piled rafts with 4, 12 and 16 piles and unpiled raft. Here piled rafts with more piles tend to show higher initial stiffness and greater load-bearing capacities. In contrast to an unpiled raft, which can support a load of 7.5kN with a settlement of 40mm, piled rafts with 4, 12, and 16 piles exhibit load-bearing capacities of approximately 8kN, 9.5kN and 12.8kN, respectively.

- 3.7 Effect of raft thickness

The variation in raft thickness holds substantial influence over the load-settlement characteristics of piled raft foundations. Thicker rafts inherently bestow greater load-carrying capacity, diminished settlements, and stiffer response. Illustrated through the array of load-settlement curves corresponding to various raft thicknesses, ranging from 0.25m to 1.50m (depicted in Fig. 11), this parameter significantly shapes the performance of the foundation. A noticeable trend becomes evident: with an increase in raft thickness, measuring 0.25m, 0.50m, 0.75m, 1m, 1.25m and 1.5m, there is a corresponding rise in loadbearing capacity. Yet, it is notable that thicker rafts exhibit a comparatively higher initial settlement, a

Fig. 11 Effect of raft thickness

phenomenon attributed to the self-weight of the raft [18].

Thus, the magnitude of raft thickness becomes a pivotal consideration in optimizing the structural design and performance of piled raft foundations across diverse geotechnical scenarios.

- 4 Conclusions

An investigation using a 3D finite element approach was carried out to comprehend the performance of substantial piled raft foundations on sandy soil. Derived from the outcomes of the finite element analysis, the subsequent deductions can be made:

- Increased pile length enhances both load-bearing capacity and stiffness while reducing the settlement. It is worth noting that there is a significant increase of almost 60% in the load-bearing capacity when the length of the pile increases from 10m to 20m.

- Load-settlement curves revealed a significant correlation, indicating that higher values of Poisson’s ratio were associated with increased load-bearing capacities. Notably, this augmentation ranged from 1.5% to 2.5% within the specified range of Poisson’s ratio, which was between 0.2 and 0.4.

- Elevated friction angles enhance skin friction and end-bearing capacity, leading to elevated load-bearing capacity and reduced settlements in load-settlement curves. The angle shift from 25° to 30° resulted in a 10% load capacity increase, followed by substantial rises of 16.5% at 35°and 21.5% at 40°. This pattern demonstrates diminishing percentage enhancements as friction angles increase, with subsequent adjustments yielding improvements of 10%, 5.5%, and 4.4% respectively.

- In comparison to a reference diameter of 0.25m, the load-bearing capacity exhibits constant and significant percentage improvements with each successive 0.25m increment. These increments lead to approximate enhancements of 83%, 200%, and 315% correspondingly. Nevertheless, as the diameter increases by 0.25m in each successive stage, the succeeding percentage growth gradually decreases: commencing at 83%, followed by 64%, and ultimately reaching 38%.

- An increase in elastic modulus from 15MPa to 50MPa results in a stiffer soil, enabling it to resist applied loads more effectively. This leads to a typical increase in the foundation’s load capacity, with observed percentage increases ranging from 16% to 4.3% for every 5MPa increase.

- The number of piles in a piled raft foundation significantly influences its load-carrying capacity and settlement behaviour. Increasing the number of piles results in improved load distribution leading to higher load-bearing capacities.

- This is evident in load-settlement curves, where piled rafts with more piles exhibit greater initial stiffness and increased load-bearing capacities compared to unpiled rafts. For instance, with respect to an unpiled raft with a settlement of 40mm, the bearing capacity of piled rafts with 4, 12, and 16 piles increases by approximately 6.6%, 26.67%, and 70.67%, respectively.

- The impact of raft thickness on piled raft foundations is significant. Thicker rafts enhance load-carrying capacity and stiffness while reducing settlements, although they may initially experience higher settlements due to self-weight. The current research showed a 200% load capacity increase as raft thickness progressed from 0.25m to 1.5m. Nonetheless, it is crucial to carefully optimise the thickness increment to ensure cost-effectiveness.

It is crucial to acknowledge that the benefits of increasing most parameters tend to diminish once specific thresholds are reached. This underscores the importance of adopting a careful approach when making adjustments to these parameters in order to achieve optimal load-bearing capacity, while also ensuring efficiency and cost-effectiveness.

- References

1. 	Katzenbach R, Arslan U, Moormann C (2000) Piled raft foundation projects in Germany. Des Appl raft Found 323–391. https://doi.org/10.1680/daorf.27657.0013

2. 	Poulos HG (2005) Piled Raft and Compensated Piled Raft Foundations for Soft Soil Sites. Adv Des Test Deep Found 214–234

3. 	Butterfield R, Banerjee PK (1971) The Elastic Analysis of Compressible Piles and Pile Groups. Geotechnique 21:43–60. https://doi.org/10.1680/geot.1971.21.1.43

4. 	Poulos HG (1994) An approximate numerical analysis of pile–raft interaction. Int J Numer Anal Methods Geomech 18:73–92. https://doi.org/10.1002/nag.1610180202

5. 	Hooper JA (1973) Observations on the Behaviour of a Piled-Raft Foundation on London Clay. Proc Inst Civ Engrs 55:855–877. https://doi.org/10.1680/iicep.1973.4144

6. 	Burland JB, Broms BB, de Mello VFB (1977) Behaviour of foundations and structures. Proc Int Conf Soil Mech Found Eng Tokyo, Japan July 10-15, 1977 2:495–536

7. 	Randolph M. (1994) Design methods for pile groups and piled rafts. In: Proc. XIII ICSMFE: 5, New Delhi, India. pp 61–82

8. 	de Sanctis L, Mandolini A (2006) Bearing Capacity of Piled Rafts on Soft Clay Soils. J Geotech Geoenvironmental Eng 132:1600–1610. https://doi.org/10.1061/(asce)1090-0241(2006)132:12(1600)

9. 	Lee J, Park D, Park D, Park K (2015) Estimation of load-sharing ratios for piled rafts in sands that includes interaction effects. Comput Geotech 63:306–314. https://doi.org/10.1016/j.compgeo.2014.10.014

10. 	Butterfield R, Banerjee PK (1971) The Problem of Pile Group-Pile Cap Interaction. Géotechnique 135–142

11. 	Kuwabara F (1989) An elastic analysis for piled raft foundations in a homogeneous soil. Japanese Soc Soil Mech Found Eng 29:82–92

12. 	Ta LD, Small JC (1996) Analysis of piled raft systems in layered soils. Int J Numer Anal Methods Geomech 20:57–72. https://doi.org/10.1002/(sici)1096-9853(199601)20:1<57::aid-nag807>3.0.co;2-0

13. 	Unsever YS, Matsumoto T, Özkan MY (2015) Numerical analyses of load tests on model foundations in dry sand. Comput Geotech 63:255–266. https://doi.org/10.1016/j.compgeo.2014.10.005

14. 	Pandey G, Mourya VK, Patel D, et al (2024) Load sharing behaviour in piled-raft foundations over sand and clay: An experimental investigation. Res Eng Struct Mater x:1–26. https://doi.org/10.17515/resm2023.41me0714rs

15. 	Rasouli H, Saeedi Azizkandi A, Baziar MH, et al (2015) Centrifuge modeling of non-connected piled raft system. Int J Civ Eng 13:114–123

16. 	Viggiani C, Mandolini A, Russo G (2012) Piles and Pile Foundations

17. 	Vakili R (2015) Load Sharing Mechanism of Piled-Raft Foundation in Sand

18. 	Sinha A (2013) 3-D Modeling of Piled Raft Foundation. 235

19. 	Bowles JE (1987) Elastic foundation settlements on sand deposits. J Geotech Eng 113:846–860. https://doi.org/10.1061/(ASCE)0733-9410(1989)115:3(427)